mtu

Murex Welding Products Limited, Hertford Road, Waltham C ross , Hertfordshire EN8 7RP Website : www .murexwelding .co .uk

The Murex Transarc "Open Circuit Voltage Reducer" is designed for use when Manual Metal Arc welding up to 400A with either a DC (rectifier or generator) or AC (transformer) power sources. It provides a low voltage at the electrode holder (<24Vdc) when welding is not in progress. This reduces the risk of electric shock when operating in areas of increased hazard.

In-Line MMA Open Circuit Voltage Reducer Meets all new ENs and safety legislation

t	AC or DC operation

Safe 24V de at the electrode when arc not struck No auxiliary power required to operate

Sealed wall mountable metal enclosure

Specification

.lb.;

Unique stall protection circuit switches off power if electrode frozen in weld

Voltage reduced to 24V within 0.3 seconds of breaking the arc

Power Source OVC: Polarity:

Rating:

1OOV RMS or Average t-1\0..X.

AC or DC

400A at 35% duty (10 min)

300A at 60%

250A 100%

Reset Times: Standards: Dimensions: Weight (net):

T1:0-2sec, T2: 0.3-2secs EN609741, EN50199, IP23

300x300x120mm

11.5kg

| Reduced OCV: Arc snaps Electrode momentarily out touched to work Electrode t pulled away Electrode from work touched to t Ele<:trode work gain, Normal Normal Movement strik ing the arc Welding Welding + r--- - --;·...... .................. ... ....... yln; Tl I ·--- -- -- ·-·_.. -·-_·_- _._.. .. r-- -- -_- _··_·_···__··_···--· l l If arc: not struck OCV reduced to within time T1 24Vdc after lime T2 OCV reduced lo 24Vdc: T1 variable 0 to 2 sees |  |
| --- | --- |
| T2 va · ble 0• | .3 to 2 sees |

Operation

24Vdc max

Part No:	1415590

Transarc OCVR AC/DC

Open _.Circuit Voltage Redu·cer

Introduction

The Murex Transarc "Open Circuit Voltage Reducer" is designed for use when Manual Metal Arc welding up to 400A with either DC (rectifier) or AC (transformer) power sources . It provides a low voltage at the electrode holder (<24Vdc) when welding is not in progress. This reduces the risk of electric shock when operating in areas of increased hazard.

Manufactured in a fully enclosed weatherproof case the unit should be vertically mounted on  a  suitable  wall  or bulkhead (cables and connections under). No auxiliary or independent power is required to run the  unit  as  it is powered automatically from the open circuit or arc voltage. A red indicator light on the cover panel is illuminated when reduced OCV is present at the electrode holder .

An exclusive monitoring circuit in the Transarc OCVR senses if the electrode is stalled or stuck on the work and automatically disconnects welding output.

Specification

Power  Source  OVC: Polarity :

Rating:

Reduced OCV:

1OOV RMS or Average AC or DC

400A at 35% duty (10 min)

300A at 60%

250A at 100%

24Vdc max

Reset Times: Standards: Dimensions: Weight (net) :

T1: 0-2sec, T2: 0.3-2secs EN609741, EN50199 , IP23

300x300x120mm 11.5kg

Installation/Connection

It is recommended that the unit be vertically mounted on a convenient wall or bulkhead with the inlets and outlets at the bottom to prevent the ingress of water.   Although the unit is weatherproof,  Murex recommends that it is protected from rain or direct water spray .  DO NOT WELD IN THE RAIN

DC Pos or Neg	AC

DC

Rectifier

or Generator

AC

Transformer

IN DC DO NOT REVERSE THE POWER SOURCE CONNECTIONS